CARY DMS-500 IR Remote Codes

Dec, 29, 2015

| All codes - to Philips RC-5 standard |  |
| --- | --- |

|  | Key Printing |  | Function | System | Command |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | POWER |  | POWER ON / OFF | 0x10 | 0x0C |  |  |  |
|  | UP |  | NAVIGATION UP | 0x10 | 0x35 |  |  |  |
|  | DOWN |  | NAVIGATION DOWN | 0x10 | 0x36 |  |  |  |
|  | LIFT |  | NAVIGATION LIFT | 0x10 | 0x1F |  |  |  |
|  | RIGHT |  | NAVIGATION RIGHT | 0x10 | 0x2A |  |  |  |
|  | ENTER |  | ENTER | 0x10 | 0x21 |  |  |  |
|  | MENU |  | MENU | 0x10 | 0x28 |  |  |  |
|  | BRIGHT |  | BRIGHT | 0x10 | 0x29 |  |  |  |
|  | MUTE |  | MUTE | 0x10 | 0x2D |  |  |  |
|  | - VOLUME |  | VOLUME DOWN | 0x10 | 0x11 |  |  |  |
|  | VOLUME + |  | VOLUME UP | 0x10 | 0x10 |  |  |  |
|  | 1 |  | NUMBER KEY PAD 1 | 0x10 | 0x0E |  |  |  |
|  | 2 |  | NUMBER KEY PAD 2 | 0x10 | 0x0B |  |  |  |
|  | 3 |  | NUMBER KEY PAD 3 | 0x10 | 0x1E |  |  |  |
|  | 4 |  | NUMBER KEY PAD 4 | 0x10 | 0x3E |  |  |  |
|  | 5 |  | NUMBER KEY PAD 5 | 0x10 | 0x01 |  |  |  |
|  | 6 |  | NUMBER KEY PAD 6 | 0x10 | 0x02 |  |  |  |
|  | 7 |  | NUMBER KEY PAD 7 | 0x10 | 0x03 |  |  |  |
|  | 8 |  | NUMBER KEY PAD 8 | 0x10 | 0x0F |  |  |  |
|  | 9 |  | NUMBER KEY PAD 9 | 0x10 | 0x04 |  |  |  |
|  | 0 |  | NUMBER KEY PAD 0 | 0x10 | 0x05 |  |  |  |
|  | *.-/ |  | NUMBER KEY PAD *.-/ | 0x10 | 0x06 |  |  |  |
|  | PLAY MODE |  | PLAY MODE | 0x10 | 0x0A |  |  |  |
|  | DELETE |  | DELETE | 0x10 | 0x07 |  |  |  |
|  | ADD |  | ADD | 0x10 | 0x3C |  |  |  |
|  | WXYZ |  | WXYZ | 0x10 | 0x24 |  |  |  |
|  | # OPTICAL |  | OPTICAL INPUT | 0x10 | 0x3D |  |  |  |
|  | # COAXIAL 1 |  | COAXIAL 1 INPUT | 0x10 | 0x38 |  |  |  |
|  | # COAXIAL 2 |  | COAXIAL 2 INPUT | 0x10 | 0x14 |  |  |  |
|  | # BLUETOOTH |  | BLUETOOTH INPUT | 0x10 | 0x15 |  |  |  |
|  | # USB / SD CARD |  | USB/ SD CARD INPUT | 0x10 | 0x16 |  |  |  |
|  | # AES/EBU |  | AES/EBU INPUT | 0x10 | 0x17 |  |  |  |
|  | # SRC |  | SRC | 0x10 | 0x3B |  |  |  |
|  | # HOME |  | HOME | 0x10 | 0x1A |  |  |  |
|  | # QUEUE |  | QUEUE | 0x10 | 0x33 |  |  |  |
|  | # INTERNET |  | INTERNET INPUT | 0x10 | 0x13 |  |  |  |
|  | # NETWORK |  | NETWORD INPUT | 0x10 | 0x22 |  |  |  |
|  | # NOW PLAYING |  | NOW PLAYING | 0x10 | 0x23 |  |  |  |